Custom JMEDIA / JMISC Carrier Board

Design Reference — Based on Arduino ASX00085 Breakout Carrier

JMEDIA → RPi Camera 15-pin FPC  •  JMISC → 2×20 2.54mm Header  •  PCA9306 I2C Level Shifting

Arduino released the ASX00085 UNO Breakout Carrier on 02/04/2026. This document uses its verified dimensions, connector positions, and pin mappings as the reference for your custom board. The only functional differences are: (1) JMEDIA MIPI CSI-2 lanes are routed directly to a 15-pin FPC connector for the Raspberry Pi Camera Module rather than left as NC, and (2) a PCA9306 level-shifter bridges the 1.8V CCI I2C bus to 3.3V for the camera.

# 1  Board Concept vs Arduino ASX00085

The table below highlights every difference and similarity between the official Arduino breakout carrier and your custom board. Dimensions and connector positions are taken directly from the ASX00085 datasheet (pages 13–14).

| Feature | Arduino ASX00085 Breakout Carrier | Your Custom Board (This Design) |
| --- | --- | --- |
| SKU / Name | ASX00085 • UNO Breakout Carrier | Custom — PulseQ JMEDIA Carrier |
| JMEDIA connector | 2×30 1.27mm SMT male (mates to Uno Q) | 2×30 1.27mm SMT male (identical) |
| JMISC connector | 2×30 1.27mm SMT male (mates to Uno Q) | 2×30 1.27mm SMT male (identical) |
| JMEDIA output | 2×20 2.54mm male header (J14) — GPIO/I2C/power only; all MIPI pins NC | 15-pin FPC 1mm (SFW15R-2STE1LF) — direct MIPI CSI-2 to RPi camera |
| JMISC output | 2×20 2.54mm male header (J15) — MCU GPIO, audio, OPAMP | 2×20 2.54mm male header (J15) — identical to ASX00085 |
| Level shifting | None (JMEDIA I2C stays at 1.8V) | PCA9306 on CCI I2C (1.8V → 3.3V for camera) |
| Board size (ref) | 107.6 × 53.34 mm | ~107.6 × 60 mm (slight height increase for FPC) |
| JMEDIA–JMISC spacing | 56.83 mm center-to-center | 56.83 mm (must match exactly) |
| JMEDIA connector X offset | 9.97 mm from left edge | 9.97 mm (must match exactly) |
| MIPI lanes broken out? | No — all NC on J14 | Yes — CSI0 Lane 0, Lane 1, Clock → FPC |
| Target use | General prototyping, debug | RPi Camera integration + PulseQ |

# 2  JMEDIA Connector — Complete Pin Routing

The JMEDIA connector on the ASX00085 routes only GPIO_20, GPIO_21, and CCI I2C signals to the J14 2×20 header — all MIPI lanes are NC. Your board routes the MIPI CSI-2 lanes directly to the RPi camera FPC connector (J3). All other routing mirrors ASX00085 J14.

KEY DIFFERENCE: The ASX00085 leaves JMEDIA pins 3,4,5,6,9,10,12,15,17,21,23,27,29,33,35,39–48,52,54 as NC. Your board connects pins 21,23,27,29,33,35 directly to the FPC connector for the camera's MIPI CSI-2 data and clock lanes.

CONNECTOR SPACING: JMEDIA is positioned 9.97mm from the left board edge. JMISC is 56.83mm to the right (center-to-center). These dimensions are from the ASX00085 mechanical drawing (page 14) and must be replicated exactly.

| JMEDIA Pin | Signal / ASX00085 Function | Type | Your Board Routing | FPC Pin | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | GND | PWR | Board GND | — |  |
| 2 | GND | PWR | Board GND | — |  |
| 3–6 | NC | — | NC (MIPI DSI — unused) | — | DSI display, not needed for RPi camera |
| 7 | GND | PWR | Board GND | — |  |
| 8 | GND | PWR | Board GND | — |  |
| 9–10 | NC | — | NC (MIPI DSI) | — |  |
| 11 | GND | PWR | Board GND | — |  |
| 12 | NC | — | NC | — |  |
| 13 | GND | PWR | Board GND | — |  |
| 14 | GND | PWR | Board GND | — |  |
| 15 | NC | — | NC | — |  |
| 16 | SOC_CAM_MCLK0 (GPIO_20) | 1.8V DIG | → J15/J14 header pin + FPC pin 12 | 12 | CAM_IO0 power-enable (active HIGH, 1.8V — check camera tolerance) |
| 17 | NC | — | NC | — |  |
| 18 | SOC_CAM_MCLK1 (GPIO_21) | 1.8V DIG | → J14 header pin only | — | Optional GPIO |
| 19 | GND | PWR | Board GND | — |  |
| 20 | GND | PWR | Board GND | — |  |
| 21 | CSI0_C0_LN0_M | MIPI D− | → FPC pin 2 direct (100Ω diff) | 2 | CSI Data Lane 0 − |
| 22 | CCI_I2C_SDA1 (GPIO_29) | 1.8V I2C | → J14 header only (CCI bus 1) | — | Camera 1 I2C — not used for RPi |
| 23 | CSI0_B0_LN0_P | MIPI D+ | → FPC pin 3 direct (100Ω diff) | 3 | CSI Data Lane 0 + |
| 24 | CCI_I2C_SCL1 (GPIO_30) | 1.8V I2C | → J14 header only | — |  |
| 25 | GND | PWR | Board GND / FPC pin 4 | 4 |  |
| 26 | GND | PWR | Board GND | — |  |
| 27 | CSI0_B1_LN1_M | MIPI D− | → FPC pin 5 direct (100Ω diff) | 5 | CSI Data Lane 1 − |
| 28 | NC | — | NC (CSI1 lane — unused) | — |  |
| 29 | CSI0_A1_LN1_P | MIPI D+ | → FPC pin 6 direct (100Ω diff) | 6 | CSI Data Lane 1 + |
| 30 | NC | — | NC | — |  |
| 31 | GND | PWR | Board GND / FPC pin 7 | 7 |  |
| 32 | GND | PWR | Board GND | — |  |
| 33 | CSI0_A0_CLK_M | MIPI CLK− | → FPC pin 8 direct (100Ω diff) | 8 | CSI Clock − |
| 34 | NC | — | NC (CSI1) | — |  |
| 35 | CSI0_NC_CLK_P | MIPI CLK+ | → FPC pin 9 direct (100Ω diff) | 9 | CSI Clock + |
| 36 | NC | — | NC | — |  |
| 37 | GND | PWR | Board GND / FPC pin 10 | 10 |  |
| 38 | GND | PWR | Board GND | — |  |
| 39–48 | NC | — | NC (CSI0 L2/L3, CSI1) | — | Leave floating — RPi camera is 2-lane only |
| 49 | GND | PWR | Board GND / FPC pin 13 | 13 |  |
| 50 | GND | PWR | Board GND | — |  |
| 51 | CCI_I2C_SCL0 (GPIO_23) | 1.8V I2C | → PCA9306 SCL1 → FPC pin 15 | 15 | LEVEL SHIFTED 1.8V→3.3V |
| 52 | NC | — | NC | — |  |
| 53 | CCI_I2C_SDA0 (GPIO_22) | 1.8V I2C | → PCA9306 SDA1 → FPC pin 14 | 14 | LEVEL SHIFTED 1.8V→3.3V |
| 54 | NC | — | NC | — |  |
| 55 | GND | PWR | Board GND | — |  |
| 56 | GND | PWR | Board GND | — |  |
| 57 | VIN | PWR IN | Board VIN rail | — | 7–24V raw |
| 58 | +3V3 | PWR OUT | → 3.3V power plane, VREF2 PCA9306, R1/R2 pull-ups | — |  |
| 59 | VIN | PWR IN | Board VIN rail | — |  |
| 60 | +3V3 | PWR OUT | → 3.3V power plane | — |  |

MIPI ROUTING RULE: Route lanes 0/1 and clock as 100Ω differential pairs. Length-match all three pairs to within ±2mm. Place FPC connector J3 as close as possible to JMEDIA J1 — target < 20mm trace length. No vias, no width changes, no stubs.

# 3  JMISC Connector — Complete Pin Routing

Your JMISC output is identical to the ASX00085 J15 2×20 header. The table below maps every JMISC pin to the output header. JMISC pin 57 (+1V8 OUT) is also used as VREF1 for the PCA9306 level-shifter.

| JMISC Pin | Signal / ASX00085 Function | Type | Your Board Routing |
| --- | --- | --- | --- |
| 1 | MCU_PSSI_D0 / PC6 | 3.3V DIG | → 2×20 header (J15) pin 1 |
| 2 | MCU_SDMMC1_CMD / PD2 | 3.3V DIG | → J15 pin 2 |
| 3 | MCU_PSSI_D1 / PC7 | 3.3V DIG | → J15 pin 3 |
| 4 | MCU_TRACE_CLK / PE2 | 3.3V DIG | → J15 pin 4 |
| 5 | MCU_PSSI_D2 / PC8 | 3.3V DIG | → J15 pin 5 |
| 6 | MCU_TRACE_D0 / PE3 | 3.3V DIG | → J15 pin 6 |
| 7 | MCU_PSSI_D3 / PC9 | 3.3V DIG | → J15 pin 7 |
| 8 | MCU_TRACE_D2 / PE5 | 3.3V DIG | → J15 pin 8 |
| 9 | MCU_PSSI_D4 / PE4 | 3.3V DIG | → J15 pin 9 |
| 10 | MCU_TRACE_D3 / PE6 | 3.3V DIG | → J15 pin 10 |
| 11 | MCU_PSSI_D5 / PI4 | 3.3V DIG | → J15 pin 11 |
| 12 | MCU_PE7 / PE7 | 3.3V DIG | → J15 pin 12 |
| 13 | MCU_PSSI_D6 / PI6 | 3.3V DIG | → J15 pin 13 |
| 14 | MCU_PE8 / PE8 | 3.3V DIG | → J15 pin 14 |
| 15 | MCU_PSSI_D7 / PI7 | 3.3V DIG | → J15 pin 15 |
| 16 | MCU_I2C4_SCL / PF14 | 3.3V I2C | → J15 pin 16 |
| 17 | MCU_PSSI_PDCK / PD9 | 3.3V DIG | → J15 pin 17 |
| 18 | MCU_I2C4_SDA / PF15 | 3.3V I2C | → J15 pin 18 |
| 19 | MCU_PSSI_RDY / PI5 | 3.3V DIG | → J15 pin 19 |
| 20 | MCU_OPAMP1_VOUT / PA3 | ANALOG | → J15 pin 20 |
| 21 | MCU_PSSI_DE / PD8 | 3.3V DIG | → J15 pin 21 |
| 22 | MCU_OPAMP1_VINP / PA0 | ANALOG | → J15 pin 22 |
| 23 | MCU_MCO / PA8 | 3.3V DIG | → J15 pin 23 (note: ASX00085 J15 has this at pin 26) |
| 24 | MCU_OPAMP1_VINM / PA1 | ANALOG | → J15 pin 24 |
| 25 | MCU_CRS_SYNC / PA10 | 3.3V DIG | → J15 pin 25 |
| 26 | GND | PWR | → GND |
| 27 | GND | PWR | → GND |
| 28 | EAR_P_R | ANALOG | → J15 pin (audio) |
| 29 | MIC2_INP | ANALOG | → J15 pin (audio) |
| 30 | EAR_M_R | ANALOG | → J15 pin (audio) |
| 31 | MIC2_INM | ANALOG | → J15 pin (audio) |
| 32 | LINEOUT_P | ANALOG | → J15 pin (audio) |
| 33 | MIC2_BIAS | ANALOG | → J15 pin (audio) |
| 34 | LINEOUT_M | ANALOG | → J15 pin (audio) |
| 35 | GND | PWR | → GND |
| 36 | HPH_L | ANALOG | → J15 pin (audio) |
| 37 | SOC_GPIO_0_SE0 | 1.8V DIG | → J15 MPU GPIO |
| 38 | HPH_R | ANALOG | → J15 pin (audio) |
| 39 | SOC_GPIO_1_SE0 | 1.8V DIG | → J15 MPU GPIO |
| 40 | HPH_REF | ANALOG | → J15 pin (audio) |
| 41–52 | SOC_GPIO_2/3/86/82/18/28 + SE0 | 1.8V DIG | → J15 MPU GPIOs (match ASX00085 J15 layout) |
| 53 | +3V3 | PWR OUT | → 3.3V power plane |
| 54 | +5V USB | PWR OUT | → J15 pin (5V out) |
| 55 | +3V3 | PWR OUT | → 3.3V power plane |
| 56 | +5V USB | PWR OUT | → J15 pin |
| 57 | +1V8 | PWR OUT | → VREF1 of PCA9306 + J15 pin |
| 58 | GND | PWR | → GND |
| 59 | VCOIN | PWR IN | → J15 pin (RTC battery) |
| 60 | VBAT | PWR OUT | → J15 pin (+3.8V) |

# 4  PCA9306 Level-Shifter Wiring

## 4.1  Why needed

JMEDIA CCI_I2C_SDA0 (pin 53) and CCI_I2C_SCL0 (pin 51) are at 1.8V. The RPi camera FPC pins 14 (SDA) and 15 (SCL) expect 3.3V (pulled up on the camera module). Direct connection damages the QRB2210 SoC.

## 4.2  Connections

- PCA9306 VREF1 (pin 2) → +1.8V from JMISC pin 57

- PCA9306 VREF2 (pin 7) → +3.3V from JMEDIA pin 58

- PCA9306 EN (pin 8) → tie to VREF2 (+3.3V) to keep always enabled

- PCA9306 SCL1 (pin 3) → JMEDIA pin 51 (CCI_I2C_SCL0, 1.8V side)

- PCA9306 SDA1 (pin 4) → JMEDIA pin 53 (CCI_I2C_SDA0, 1.8V side)

- PCA9306 SDA2 (pin 5) → FPC J3 pin 14 (CAM_SDA, 3.3V side)

- PCA9306 SCL2 (pin 6) → FPC J3 pin 15 (CAM_SCL, 3.3V side)

- R3, R4 (4.7kΩ) → pull SDA1/SCL1 to +1.8V

- R1, R2 (10kΩ) → pull SDA2/SCL2 to +3.3V (check if camera module already has pull-ups)

- C1 (100nF) → VREF1 to GND, place within 0.5mm of pin 2

- C2 (100nF) → VREF2 to GND, place within 0.5mm of pin 7

# 5  Bill of Materials

| Ref | Mfr | Part Number | Description | KiCad/Source |
| --- | --- | --- | --- | --- |
| J1 | Samtec | FTSH-130-01-L-DV | Male SMT 2×30 1.27mm — JMEDIA mate | Ultra Librarian / Samtec.com |
| J2 | Samtec | FTSH-130-01-L-DV | Male SMT 2×30 1.27mm — JMISC mate | Ultra Librarian / Samtec.com |
| J3 | Amphenol FCI | SFW15R-2STE1LF | 15-pin FPC 1mm R/A Top — RPi Camera | SnapEDA / Ultra Librarian |
| J4 | Generic | 2×20 2.54mm THT male header | JMISC output breakout (= ASX00085 J15) | KiCad built-in / local |
| J5 | Generic | 2×20 2.54mm THT male header | JMEDIA output (GPIO/power/I2C only) | KiCad built-in / local |
| U1 | Texas Instruments | PCA9306DCTR | I2C level shifter 1.8V↔3.3V, SSOP-8 | KiCad built-in + SnapEDA |
| R1 | Generic | 10kΩ 0402 | Pull-up SDA 3.3V side | KiCad Device:R |
| R2 | Generic | 10kΩ 0402 | Pull-up SCL 3.3V side | KiCad Device:R |
| R3 | Generic | 4.7kΩ 0402 | Pull-up SDA 1.8V side | KiCad Device:R |
| R4 | Generic | 4.7kΩ 0402 | Pull-up SCL 1.8V side | KiCad Device:R |
| C1 | Generic | 100nF 0402 | Decoupling PCA9306 VREF1 (1.8V) | KiCad Device:C |
| C2 | Generic | 100nF 0402 | Decoupling PCA9306 VREF2 (3.3V) | KiCad Device:C |
| C3 | Generic | 10µF 0805 | Bulk 3.3V rail | KiCad Device:C |

# 6  Mechanical Dimensions

All critical dimensions are taken from the ASX00085 datasheet pages 13–14. The connector center-to-center spacing (56.83mm) and left-edge offset (9.97mm) must be replicated with < 0.05mm tolerance.

| Dimension | Value | Notes |
| --- | --- | --- |
| Board width | 107.6 mm (ASX00085) → your board: same or slightly wider if needed | Match exactly to ensure connectors align with Uno Q |
| Board height | 53.34 mm (ASX00085) → suggest ~60 mm | Extra height for FPC connector and routing |
| JMEDIA left-edge offset X | 9.97 mm | Left edge of JMEDIA connector from board left edge |
| JMISC position X | 9.97 + 56.83 = 66.80 mm | Left edge of JMISC connector |
| JMEDIA ↔ JMISC center-to-center | 56.83 mm | CRITICAL — must match Uno Q exactly |
| JMEDIA connector body length | 38.1 mm (2×30 at 1.27mm = 37.73mm + housing) | Leave 1mm clearance each side |
| JMISC connector body length | 50.8 mm | Same calculation |
| 2×20 header pitch | 2.54mm — 20 rows × 2.54 = 50.8mm length | Place in area right of JMISC |
| FPC connector (J3) position | Suggest: above JMEDIA area, Y ≈ 45–55mm | Short traces to JMEDIA MIPI pins (< 20mm ideal) |
| Mounting holes | Ø3.2mm — match ASX00085 pattern | 4× holes as per datasheet mechanical drawing |
| Board thickness | Standard 1.6 mm FR4 | Impedance target: 100Ω diff, adjust trace W/S per stackup |

CRITICAL: The JMEDIA ↔ JMISC center-to-center distance is 56.83mm as shown in ASX00085 page 14. Your PCB layout must place the two FTSH-130 connectors at exactly this spacing or the board will not mate with the Uno Q.

# 7  PCB Layout Guidelines

## 7.1  Layer Stack Recommendation

- 4-layer board: Signal Top / GND / PWR / Signal Bottom

- Target 100Ω differential impedance for MIPI traces — typical for 0.15mm trace, 0.15mm gap on 1.6mm FR4 with standard 4-layer stackup (verify with your PCB fab's impedance calculator)

## 7.2  Placement Priority

- Step 1: Place JMEDIA (J1) at X=9.97mm from left edge — this is the fixed reference point

- Step 2: Place JMISC (J2) at X=9.97+56.83=66.80mm from left edge

- Step 3: Place FPC connector (J3) as close as possible to J1, ideally above it within 15–20mm

- Step 4: Place PCA9306 (U1) between J1 and J3 on the 1.8V side

- Step 5: Place 2×20 headers J4 (JMISC) and J5 (JMEDIA general) to the right of JMISC

## 7.3  MIPI Routing Rules

- Route all MIPI pairs (L0−/L0+, L1−/L1+, CLK−/CLK+) on the same layer without vias

- Keep intra-pair length match to < 0.1mm

- Keep inter-pair length match to < 2mm (all three pairs the same length)

- Maintain 3× trace-width clearance from other signals and power planes

- No right-angle bends — use 45° or curved routing

- Add GND stitching vias alongside MIPI traces every 3–5mm

## 7.4  Power Planes

- +3.3V from JMEDIA pin 58/60 — use a plane pour in the JMEDIA area

- +1.8V from JMISC pin 57 — small pour for PCA9306 VREF1 and pull-ups

- GND — full plane on Layer 2 (inner), stitched with vias at all connectors

## 7.5  Design Rule Check

- Minimum trace width: 0.1mm for signal, 0.2mm for power

- Minimum clearance: 0.1mm general, 0.15mm for MIPI differential pairs

- Verify FTSH-130 footprint land pattern against Samtec drawing before submitting gerbers