Arduino Uno Q — Daughter Board

Raspberry Pi Camera Interface Reference

JMEDIA Connector Signal Mapping  •  KiCad Footprints & Symbols  •  Level-Shifter Wiring

Scope: This document covers the complete signal mapping from a Raspberry Pi Camera Module (15-pin CSI FPC) through the daughter board to the Arduino Uno Q JMEDIA (B2) connector via two mating Samtec FTSH-130-01-L-DV headers. It includes the full pin-to-pin connection table, level-shifting design for the CCI I2C bus, all component part numbers, and KiCad symbol/footprint locations.

# 1  Signal Mapping: RPi Camera → JMEDIA

## 1.1  Overview

The RPi Camera Module uses a 2-lane MIPI CSI-2 interface over a 15-pin, 1mm-pitch FPC connector. The Arduino Uno Q JMEDIA connector exposes 4-lane MIPI CSI-2 for two cameras (CSI0 and CSI1) at 1.8V. The camera connects to CSI0 lanes 0 & 1, the CSI0 clock, and the CCI (Camera Control Interface) I2C bus.

⚠  WARNING: All JMEDIA signals operate at 1.8V. The MIPI differential lanes are self-biased and voltage-compatible, but the CCI I2C bus (SDA/SCL) on the camera side is pulled up to 3.3V on standard RPi cameras. A bidirectional level shifter (PCA9306) is REQUIRED between the camera I2C lines and JMEDIA.

ℹ  NOTE: The RPi camera's CAM_IO0 (pin 12) is typically used as an active-HIGH power-enable signal. On the Uno Q, SOC_CAM_MCLK0 (JMEDIA pin 16) can serve this function as a 1.8V GPIO output. CAM_IO0 on the camera is usually 3.3V tolerant, but verify for your specific camera module.

## 1.2  Pin-by-Pin Connection Table

The table below maps every pin of the RPi Camera 15-pin connector to the corresponding pin on JMEDIA. Colour coding: grey = ground, yellow = requires attention, green = via level-shifter.

| RPi Pin | RPi Signal | Dir | JMEDIA Pin | JMEDIA Signal | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | GND | — | 1,2 | GND | Any GND on JMEDIA |
| 2 | CAM_D0_N | → | 21 | CSI0_C0_LN0_M | Lane 0 negative |
| 3 | CAM_D0_P | → | 23 | CSI0_B0_LN0_P | Lane 0 positive |
| 4 | GND | — | 25 | GND | Dedicated lane GND |
| 5 | CAM_D1_N | → | 27 | CSI0_B1_LN1_M | Lane 1 negative |
| 6 | CAM_D1_P | → | 29 | CSI0_A1_LN1_P | Lane 1 positive |
| 7 | GND | — | 31 | GND | Dedicated lane GND |
| 8 | CAM_CLK_N | → | 33 | CSI0_A0_CLK_M | Clock negative |
| 9 | CAM_CLK_P | → | 35 | CSI0_NC_CLK_P | Clock positive |
| 10 | GND | — | 37 | GND | Dedicated clock GND |
| 11 | CAM_IO1 | ↔ | 18 | SOC_CAM_MCLK1 (GPIO_21) | Optional GPIO — see note |
| 12 | CAM_IO0 | ← | 16 | SOC_CAM_MCLK0 (GPIO_20) | Power-enable (active HIGH) |
| 13 | GND | — | 49 | GND | Any JMEDIA GND |
| 14 | CAM_SDA | ↔ | 53 | CCI_I2C_SDA0 (GPIO_22) | Via PCA9306 level-shifter |
| 15 | CAM_SCL | ↔ | 51 | CCI_I2C_SCL0 (GPIO_23) | Via PCA9306 level-shifter |

ℹ  NOTE: Direction key: → = camera drives (output from camera); ← = board drives (input to camera); ↔ = bidirectional (I2C); — = power/ground.

## 1.3  Unused JMEDIA Pins

CSI0 lanes 2 & 3 (JMEDIA pins 39–48) and CSI1 (all lanes, JMEDIA pins 27–54 CSI1 group) are not connected for a single 2-lane RPi camera. Leave these signals unconnected (do not tie to GND or VCC).

# 2  Level-Shifter Design (PCA9306)

## 2.1  Why a Level Shifter is Required

The Uno Q JMEDIA CCI I2C signals (CCI_I2C_SDA0/SCL0) are driven by the Qualcomm QRB2210 SoC at 1.8V. Standard Raspberry Pi camera modules pull their SDA/SCL lines up to 3.3V via on-board resistors. Directly connecting these would violate the 1.8V I/O maximum of the QRB2210, potentially damaging the SoC.

The Texas Instruments PCA9306DCTR is the recommended solution: a dual-channel, bidirectional, open-drain compatible I2C voltage translator in an SSOP-8 (MSOP-8) package. It operates with VREF1 = 1.2V–3.3V and VREF2 = 1.8V–5.5V, with no direction pin needed.

## 2.2  PCA9306 Pin Connections

| Pin | Name | Connect To | Notes |
| --- | --- | --- | --- |
| 1 | GND | Board GND | Ground reference |
| 2 | VREF1 | 1.8V (from JMEDIA pin 58) | Low-voltage side supply (1.8V) |
| 3 | SCL1 | JMEDIA pin 51 (CCI_I2C_SCL0) | I2C clock — 1.8V domain |
| 4 | SDA1 | JMEDIA pin 53 (CCI_I2C_SDA0) | I2C data — 1.8V domain |
| 5 | SDA2 | FPC J3 pin 14 (CAM_SDA) | I2C data — 3.3V domain (camera) |
| 6 | SCL2 | FPC J3 pin 15 (CAM_SCL) | I2C clock — 3.3V domain (camera) |
| 7 | VREF2 | 3.3V (from JMEDIA pin 58/60) | High-voltage side supply (3.3V) |
| 8 | EN | Tie to VREF2 (3.3V) | Active-HIGH enable — always on |

ℹ  NOTE: Pull-up resistors are required on BOTH sides: 4.7kΩ on the 1.8V side (SDA1/SCL1 to 1.8V) and 10kΩ on the 3.3V side (SDA2/SCL2 to 3.3V). The camera module may already have 3.3V pull-ups on-board — if so, external 3.3V pull-ups on the daughter board are optional.

ℹ  NOTE: Ensure VREF2 ≥ VREF1 + 0.6V for reliable translation. Here 3.3V − 1.8V = 1.5V, which satisfies this requirement.

## 2.3  3.3V Supply Source

Use JMEDIA pin 58 or 60 (+3V3 OUT) to supply the 3.3V rail on the daughter board. This powers VREF2 of the PCA9306, the 3.3V pull-up resistors (R1, R2), and any 3.3V peripherals. Maximum current draw from this rail should be confirmed against the Uno Q datasheet.

JMEDIA pin 57 or 59 (VIN IN) is the raw 7–24V input — do NOT use this for logic power. JMEDIA pin 58/60 (+3V3 OUT) is the correct source.

# 3  Bill of Materials

All components required for the RPi camera interface section of the daughter board.

| Ref | Manufacturer | Part Number / Value | Description | Library Source |
| --- | --- | --- | --- | --- |
| J1 | Samtec | FTSH-130-01-L-DV | Male SMT header, 2×30, 1.27mm — JMEDIA mate | SnapEDA / Ultra Librarian |
| J2 | Samtec | FTSH-130-01-L-DV | Male SMT header, 2×30, 1.27mm — JMISC mate | SnapEDA / Ultra Librarian |
| J3 | Amphenol FCI | SFW15R-2STE1LF | 15-pin FPC/FFC, 1mm pitch, R/A, Top contact | SnapEDA / Ultra Librarian |
| U1 | Texas Instruments | PCA9306DCTR | Dual I2C bidirectional level shifter, SSOP-8 | KiCad built-in + SnapEDA |
| R1 | Generic | 10kΩ 0402 SMD | Pull-up, SDA — 3.3V side | KiCad Resistor library |
| R2 | Generic | 10kΩ 0402 SMD | Pull-up, SCL — 3.3V side | KiCad Resistor library |
| R3 | Generic | 4.7kΩ 0402 SMD | Pull-up, SDA — 1.8V side | KiCad Resistor library |
| R4 | Generic | 4.7kΩ 0402 SMD | Pull-up, SCL — 1.8V side | KiCad Resistor library |
| C1 | Generic | 100nF 0402 SMD | Decoupling, VREF1 (1.8V) | KiCad Capacitor library |
| C2 | Generic | 100nF 0402 SMD | Decoupling, VREF2 (3.3V) | KiCad Capacitor library |
| C3 | Generic | 10µF 0805 SMD | Bulk decoupling, 3.3V rail | KiCad Capacitor library |

# 4  KiCad Footprints & Symbols

## 4.1  Footprint & Symbol Reference Table

The table below provides the exact KiCad library path for every component. Built-in components are available immediately in KiCad 7/8. Components marked with an external source require downloading from the indicated website.

| Ref | Part Number | KiCad Footprint | KiCad Symbol | Download Source |
| --- | --- | --- | --- | --- |
| J1/J2 | FTSH-130-01-L-DV | Connector_PinHeader_1.27mm: PinHeader_2x30_P1.27mm_Vertical_SMD | Connector_Generic: Conn_02x30_Odd_Even | Ultra Librarian app.ultralibrarian.com |
| J3 | SFW15R-2STE1LF | Connector_FFC-FPC: Amphenol_SFW15R-2STE1LF_1x15-1MP_P1.00mm_Horizontal | Connector_Generic: Conn_01x15 | SnapEDA snapeda.com |
| U1 | PCA9306DCTR | Package_SO: MSOP-8_3x3mm_P0.65mm | Interface_Voltage_Level_Translator: PCA9306 | KiCad Built-in |
| R1–R4 | Resistor 0402 | Resistor_SMD:R_0402_1005Metric | Device:R | KiCad Built-in |
| C1–C3 | Capacitor 0402/0805 | Capacitor_SMD:C_0402_1005Metric Capacitor_SMD:C_0805_2012Metric | Device:C | KiCad Built-in |

## 4.2  Installing External KiCad Libraries

### 4.2.1  FTSH-130-01-L-DV (Samtec — J1 / J2)

The 60-pin Samtec FTSH-130-01-L-DV male SMT header is the key connector that physically mates with the Uno Q's FLE-130 female connectors on JMEDIA and JMISC.

- Go to: https://app.ultralibrarian.com/details/3dd5cc98-1076-11e9-ab3a-0a3560a4cccc/Samtec-Inc/FTS-130-01-L-DV

- Select KiCad format and download the ZIP file

- In KiCad: Preferences → Manage Symbol Libraries → Add .kicad_sym file

- In KiCad: Preferences → Manage Footprint Libraries → Add .pretty folder

- Alternatively search snapeda.com for FTSH-130-01-L-DV for a KiCad-native download

⚠  WARNING: The FTSH series is a standard SMT micro header at 1.27mm pitch. If the exact 30-position variant is unavailable, Samtec also provides longer headers (e.g. 2×50) that can be cut to 2×30 size. Confirm pin 1 orientation carefully — it differs between male (FTSH) and female (FLE) connectors.

### 4.2.2  SFW15R-2STE1LF (Amphenol FCI — J3)

This is the 15-pin, 1mm-pitch FPC/FFC connector used to accept the Raspberry Pi camera ribbon cable on the daughter board.

- SnapEDA: https://www.snapeda.com/parts/SFW15R-2STE1LF/Amphenol%20FCI/view-part/

- Ultra Librarian: https://app.ultralibrarian.com/details/bd7f85ff-1067-11e9-ab3a-0a3560a4cccc/Amphenol-ICC/SFW15R-2STE1LF

- KiCad built-in footprint search: Connector_FFC-FPC → filter for Amphenol SFW15R

ℹ  NOTE: The -2STE1LF suffix denotes Top contact (contacts face up). The -1STE1LF variant has Bottom contact (contacts face down). Choose based on your camera cable orientation — most RPi ribbon cables require Top contact when the FPC connector is mounted facing the camera cable insertion direction.

### 4.2.3  PCA9306DCTR (TI — U1)

The PCA9306 is included in the KiCad standard library. No external download is required.

- Symbol: Interface_Voltage_Level_Translator:PCA9306  (search 'PCA9306' in symbol editor)

- Footprint: Package_SO:MSOP-8_3x3mm_P0.65mm  (the DCTR suffix = MSOP-8 / VSSOP-8 package)

- Verify the footprint matches your ordered package: DCTR = SSOP-8 (3×3mm); DCU = VSSOP-8 (2.9×3mm)

ℹ  NOTE: SnapEDA also has PCA9306DCTR at: https://www.snapeda.com/parts/PCA9306DCTR/Texas%20Instruments/view-part/ — use this to cross-check pad dimensions if in doubt.

# 5  PCB Layout Guidelines

## 5.1  MIPI Differential Pair Routing

⚠  WARNING: MIPI CSI-2 lanes are high-speed differential signals. Violating these rules can cause image corruption, no-lock, or complete camera failure.

- Route all differential pairs (D0+/D0−, D1+/D1−, CLK+/CLK−) with 100Ω differential impedance (typically ~90–100Ω for standard FR4 at typical trace widths/spacing)

- Match lengths within each pair to <0.1mm (intra-pair skew) and match all three pairs (data + clock) to <2mm of each other (inter-pair skew)

- Maintain constant spacing within each pair — avoid any stubs, vias, or width changes on MIPI traces

- Route MIPI lanes on a single layer if possible — if vias are unavoidable, add one via per signal of the pair, never just one

- Keep MIPI traces away from switching power traces, crystals, and other high-frequency signals — maintain ≥3× trace-width clearance

- Place the FPC connector (J3) close to the JMEDIA connector (J1) to minimise trace length — ideally under 30mm total

## 5.2  I2C Routing (CCI Bus)

- SDA and SCL traces can be routed as standard single-ended signals — 50Ω characteristic impedance is adequate

- Place the PCA9306 (U1) close to J1 (JMEDIA connector) to keep 1.8V-side traces short

- Place pull-up resistors R1/R2 (3.3V side) close to J3 (FPC connector)

- Place pull-up resistors R3/R4 (1.8V side) close to U1 VREF1 side

## 5.3  Decoupling

- Place C1 (100nF) within 0.5mm of PCA9306 VREF1 pin (pin 2)

- Place C2 (100nF) within 0.5mm of PCA9306 VREF2 pin (pin 7)

- Place C3 (10µF) close to the 3.3V supply entry point on the board (JMEDIA pin 58/60)

- Add 100nF decoupling on JMISC power pins as well if using JMISC in your design

## 5.4  Connector Placement & Board Outline

- J1 (JMEDIA mate) and J2 (JMISC mate) must be precisely positioned — obtain exact centre-to-centre distance from the official Arduino Uno Q hardware files (Eagle or KiCad source)

- The official hardware design files are available from: https://store.arduino.cc/products/uno-q → Documentation tab

- The FTSH connector pin 1 marker must align with JMEDIA pin 1 — double check this against the Uno Q bottom-side silkscreen

⚠  WARNING: Connector mis-alignment by even 0.127mm (half a pitch) will prevent mating. Always verify footprint dimensions against the Samtec FTSH mechanical drawing before submitting gerbers.